Figure: Client’s IP address

Figure: Server’s IP address

HTTP REQUEST:

This is the Data Link Layer. This layer is responsible for moving frames from one hop to the next.

Ethernet II falls under Data Link Layer.

Destination MAC address: 5c:62:8b:b7:55:e1 (Server)

Source MAC address : 18:c0:4d:00:bb:ad (Client)

This is the Network Layer.

Source IP address: 192.168.1.103 (client)

Destination IP address: 103.69.149.102 (server).

TCP falls under the Transport layer. 
Source port: 7754 (Client)
Destination port: 80 (Server)

SEQ and ACK number 1, indicates the order of data transmission.
Flags: Push and  Acknowledgement

Window size: 1024, means client can receive up to data size 1024 at a time.

HTTP GET request was sent using HTTP v1.1

Host specifies the domain name

Keep-alive type connection is used

The accepted headers, languages, and encoding are also specified.

Previous cookie data is also sent with the request.

HTTP RESPONSE:

This is the Data Link Layer. This layer is responsible for moving frames from one hop to the next.

Ethernet II falls under Data Link Layer.

Destination MAC address: 18:c0:4d:00:bb:ad (Client)

Source MAC address : 5c:62:8b:b7:55:e1 (Server)

This is the Network Layer.

Source IP address: 103.69.149.102 (server)

Destination IP address: 192.168.1.103 (client)

TCP falls under the Transport layer. 
Source port: 80 (Server)
Destination port: 7754 (Client)

SEQ no. 1 ; ACK no. 436
Flags: Push and  Acknowledgement

Window size: 501, means server can receive up to data size 501 at a time.

This is HTTP Response

Version: HTTP/1.1

Status Code 308 means the client should change to new location for all future requests

Connection will be closed after this response